CITY OF MOUNT DORA FIRE DEPARTMENT

Mobile & Temporary Cooking Operations Safety Checklist

AUTHORITY & CODE REFERENCES

Enforced under the Authority Having Jurisdiction (AHJ) in accordance with:

• Florida Fire Prevention Code (8th Edition)
• NFPA 1 (2021), Section 50.8
• NFPA 101, 58, 70, 96
• Florida Statutes Chapter 633
• City of Mount Dora Code of Ordinances, Chapter 22 (Fire Prevention)

Failure to comply may result in denial of operation, removal from event, or enforcement action.

STATE PREEMPTION & LOCAL AUTHORITY

Pursuant to Florida Statute §509.102, regulation of licensing and permitting for mobile food vehicles is preempted to the State. However, the Mount Dora Fire Department retains authority to enforce fire and life safety codes, conduct inspections, and ensure compliance with all applicable fire protection requirements under the Florida Fire Prevention Code and NFPA standards.

GENERAL SAFETY AWARENESS

| Check | Requirements | Notes |
| --- | --- | --- |
|  | No public seating inside food truck |  |
|  | Minimum 10 ft clearance from buildings, exits, vehicles, combustibles |  |
|  | Fire apparatus access maintained |  |
|  | Hydrants and FDC accessible |  |
|  | Approved suppression system installed |  |
|  | Portable extinguishers per NFPA 10 |  |
|  | Staff trained in extinguisher use, fuel shutoff, leak testing |  |

## FUEL & POWER

| Check | Requirement | Notes |
| --- | --- | --- |
|  | Fuel tanks filled for full operation |  |
|  | Generators shut down before refueling |  |
|  | Equipment cooled before refueling |  |
|  | Exhaust ≥10 ft from openings and exits |  |
|  | Electrical systems meet NFPA 70 |  |

Section Pass ☐    Section Fail ☐

## PROPANE SYSTEM

| Check | Requirement | Notes |
| --- | --- | --- |
|  | Shutoff valves accessible |  |
|  | Tanks secured upright |  |
|  | Leak testing completed |  |
|  | Documentation available |  |
|  | Flexible connector installed |  |

Section Pass ☐    Section Fail ☐

## OPERATIONAL SAFETY

| Check | Requirement | Notes |
| --- | --- | --- |
|  | No unattended cooking |  |
|  | Ventilation open during operation |  |
|  | Gas valves closed when not in use |  |
|  | Grease buildup removed regularly |  |

Section Pass ☐    Section Fail ☐

## SOLID FUEL SAFETY

| Check | Requirement | Notes |
| --- | --- | --- |
|  | Fuel not stored above appliances |  |
|  | 3 ft clearance maintained |  |
|  | Ash removed daily |  |
|  | Ash stored in covered metal container |  |

Section Pass ☐    Section Fail ☐

## FINAL INSPECTION SUMMARY

Vendor: __________________________

Inspector: __________________________

Date: __________________________

Overall Pass ☐    Overall Fail ☐

“Approval for operation is contingent upon compliance with all applicable fire and life safety codes as determined by the Authority Having Jurisdiction (AHJ).”

Contact:

Michael Smith - Division Chief / Community Risk Reduction

Mount Dora Fire Department

Email: smithm@mountdora.gov

Cell: (352) 516-3667

Office: (352) 735-7140 ext. 2109